# PIC Voice Controlled X10 Interface using VRBot

by Otis Irace

(all projects below were developed using MikroC 8.2 and EasyPIC5 from Mikroelektronika.)

No warranty is implied or given.

This project uses a PIC16F688 MCU and serial interface to communicate with the VRBot Module.

The voice commands are received by the VRBot,  decoded by the PIC,  and then Bit commands are sent to  the PL-513 Power Line Interface.

After the circuit construction  is complete,  program the PIC16F688 with the C code.

Next you will need to Train the Voice Commands into the VRBot Module.  Use the 9 Pin D Comm port connected to your PC.   Set for 9600 Baud.   Place the toggle switch in “program” position.  Download VRBot GUI from http://download.tigal.com/veear/VRbotGUI-1.1.5.zip

Instructions for the GUI are available at the http://www.veear.eu/Products/VRbot.aspx website.

Power up the VRBot module when you are ready for voice training.

Use the following templates for Group Programming:

Trigger  0 – Train the word “HAL”

Group 1 –  Rooms,  examples shown below…..You may setup room commands to fit your environment.

Remember to modify the wordsets (ex. const char *WordSD1[] ) also in the HAL.c program to match each index # in the Group 1 Command List.

Group 2 -  X10 Commands……

Plug in the X10 PL-513 Power Line Interface.

On power up,   "HAL the Butler  " message should appear on the LCD.   If the VRBot Module is connected correctly,  "HAL is Ready!   " message will appear.   When the LED flashes,  HAL is scanning for Voice Command input.  The buzzer will beep once when the command is valid,  and beep 3 times when voice command is not understood.

Have fun!