X10 Touch Controller

by Drumissimo

The X10 Touch controller uses a TFT - PROTO Board from MikroElektronika, a Microchip P18F4685 Microcontroller and an X10 PSC04 PowerLine Module to

control lights and appliances.

Touch Screen pins are on PORTA

TFT Display is on PORTB and PORTD

X10 PSC04 Interface on PORTC.  See X10_Color_Touch_driver.c for connections.

Schematic is provided.

The TFT-ProtoBoard is powered by 3.3V as well as the microcontroller.  This allows fewer support components and keeps the design simple.

Buttons are arranged with Tag Names that I chose for my home operation,  but the X10 Channels are sequentially numbered,  left to right,  then row to row.

First Screen has Channels 1 – 4,  4 - 8,  Second Screen has 9 – 12,  13 - 16.

I started by developing the Touch Screen graphics using MikroE’s VisualTFT Software, V2.01.

I find the software very buggy,  and had to make some adjustments.

After auto generating the code through VisualTFT,  I had to modify the following manually:

- void Check_TP() routine.  Currently places the loop brackets in the wrong location.

- Intended to use the DrawLabel() command to refresh only the x10 Chan number and Status.  Rem’d it out,  does not refresh the labels,  just overwrites them.  You will notice the whole screen refreshing when a button is pressed.  If anyone knows a work around, please share?

- void Calibrate() – Modified the auto generated routine so that the pen targets are displayed with a delay to give you time to get oriented BEFORE before executing the TP_TFT_Calibrate_Min(); and TP_TFT_Calibrate_Max(); functions.  Swapped Cal Min and Cal Max positions (from auto generated positions) for TFT Landscape Mode.

I added for additional functionality the following:

- In VisualTFT, Created “Custom” Project Settings for P18F4685.  Set ports accordingly,  See X10_Color_Touch_Driver.c for TFT Module and Touch Panel connections.  If anyone knows how to access the Custom Settings outside VisualTFT, I’d appreciate it.

- In MikroProC, created Eprom.h. - Stores and retrieves Screen Calibration data from EEPROM.  After the initial program burn and first calibration,  a dummy variable is saved in EEPROM.  If that variable is present,  it skips the screen calibration and retrieves the saved calibration values.  If that variable is not present,  it forces a manual pen calibration.

- Added recalibrate push button on PORTE.F0 to force another screen calibration.

- X10.h – X10 code generation program.

********************************************************************

Operation:

Power up MCU.

Wait for Red Cal Screen.  Set Cal Min Values,  wait 5 secs. to accept values.

Wait for Olive Cal Screen.  Set Cal Max Values,  wait 5 secs. to accept values.

Next Red Screen will Get Calibration Values and Display on Screen.

(if you wish to recalibrate,  press PORTE.F0 push button)

To use X10 Controller,  press desired Channel Button,  then ON or OFF button.

Use Next and Previous buttons for other screens.

16 X10 Device channels are provided,  more are possible.

On reboot,  new screen calibration settings should be saved for recall,  no screen calibration required.

Added x10 Receive Data Line at Bottom of Screen………

Enhancements are always welcome.   I’m just a novice c programmer.